- If Vin = 0.5 V, Vref = 3.3 V, 12-bit ADC, what is digital output?

- To generate a 20 kHz PWM with fCLK = 72 MHz and PSC = 0, what ARR should be?

- For a 12-bit ADC with Vref = 3.3 V, what is the LSB voltage step?

- According to Nyquist, what is the minimum sampling frequency for a signal bandwidth of 15 kHz?

- What ADC value corresponds to an input voltage of 2.475 V with a 12-bit ADC and Vref = 3.3 V?

- Given fCLK = 72 MHz, PSC = 71, ARR = 999. What is fPWM?

- What value is gotten when we measure 3.3 V with Vref = 5 V and 12 bits resolutions?

- The input clock is 8 MHz. If you want an interrupt every 1 s, keeping prescaler = 7999, what should ARR be set to?

- An output signal having frequency is 10Hz and the duty cycle is 20%. How long the signal stays in HIGH level in a cycle?

- In order to scan 4 different 7-segment LEDs with 50Hz, what is the best timer interrupt period?