Great — I’ll teach every slide (1 → 53) from 03_SingleCycle-unlocked.pdf in order, in child-friendly cheat-sheet style. For each slide I will:

- explain the slide content simply (like a story for a 5-year-old),

- immediately translate any technical word into simple language, and

- keep it detailed enough to learn from.

Here we go — Slide 1 → Slide 53, one by one.

### Slide 1 — Title: Single-Cycle Datapath

This is the name of the lesson.

- Single-cycle = the computer tries to finish one instruction in one big beat (one big gulp).

- Datapath = the roads and stations inside the chip where numbers travel.
Think: a tiny toy factory where each robot does one whole job every drum beat. 🥁

### Slide 2 — Outline (what we will learn)

This slide lists the main parts we’ll study:

- Introduction (what the processor does)

- RISC-V Assembly (tiny words the computer understands)

- RV32I Datapath (a simple road map inside the brain)

- Register File (fast tiny drawers to store numbers)
Everything will be explained in story form.

### Slide 3 — Outline repeated

Same list, to remind us. Like the table of contents of a book. 📚

### Slide 4 — Introduction (1/7): Execution model

Story: The computer is a machine that remembers and acts.

- Finite state machine = “a machine that remembers which step it’s on.”

- Registers = “few very fast drawers where the computer keeps numbers.”

- Memory = “a big bookshelf that stores lots of things but is slower to reach.”

- Program Counter (PC) = “an arrow pointing to the next instruction to do.”
What it does in a loop: fetch the order (grab a page), decode (read what it says), read inputs (open drawers), execute (do math), write outputs (save back), move the arrow.
Think: reading a recipe, gathering ingredients, cooking, and saving the dish. 🍽️

### Slide 5 — Introduction (2/7): ISA (Instruction Set Architecture)

- ISA = “the promise between software and hardware” — the rulebook both agree to follow.

  - Hardware (HW) = the physical toy factory (chips).

  - Software (SW) = the recipes (programs).
ISA tells: where we can store things (drawers, shelves) and what moves we can do (add, jump, load).
It is like the language the boss and workers use so they all understand each other.

### Slide 6 — Introduction (3/7): Datapath

- Datapath = roads + stations: register file, ALU, memory paths.

- ALU = “Arithmetic Logic Unit” = a tiny calculator that can add, subtract, compare, do simple logic.

- Control = the boss that tells the roads which way to send numbers (which register, which ALU operation).
Story: A cart carries a number from a shelf, goes through a gate to the calculator, gets a new number, and returns.

### Slide 7 — Introduction (4/7): Two types of components

There are two kinds of building blocks:

- Combinational logic = “instant calculators” with no memory (they just compute an output from inputs right away). Examples: ALUs, multiplexers.

- Sequential logic = “things that remember” (like the PC, memories, register file). They hold values across beats.
Think: combinational = instant lemonade machine; sequential = a fridge that keeps lumps cold.

### Slide 8 — Introduction (5/7): Example datapath

This slide shows a simple example datapath (a picture).
Think of it as the toy factory map: where the instruction comes from, where the drawers are, how the calculator sits between them. (We’ll build the map step by step.)

### Slide 9 — Introduction (6/7): Single-cycle datapath idea

- One cycle per instruction (CPI) = every instruction completes in one clock beat.

- Clock cycle time = how long the beat must be — it must be long enough for the slowest instruction to finish.
So the whole system waits long enough for the slowest job. Analogy: everyone waits until the slowest friend finishes tying shoes before moving on.

### Slide 10 — Introduction (7/7): Pipelined datapath (preview)

- Pipelining = cut the big job into smaller stages and run many jobs overlapped, like an assembly line.

- This shortens the slowest step between storage points.
Think: instead of one robot doing everything, several robots each do a small part and pass the cart along — faster overall.

### Slide 11 — Outline (reminder)

We’ll now move into RISC-V assembly. (Little reminder slide.)

### Slide 12 — RISC-V Assembly (1/16): Assembly vs Machine code

- Assembly language = human-friendly words that represent machine actions (like add x1, x2, x3).

- Machine code = the actual 0s and 1s the chip understands.

- Assembler = the translator that turns assembly into machine code.
Think: assembly is like short recipe notes; assembler turns notes into instructions the oven understands.

### Slide 13 — RISC-V Assembly (2/16): What is a 64-bit ISA?

- 64-bit ISA = addresses and pointers are 64 bits (big numbers), and registers often hold 64 bits.

- This does not tell you how many registers or instruction size — it’s just how wide the main boxes are.
Translate: “64-bit” = drawers and labels are big enough to hold very large numbers.

### Slide 14 — RISC-V Assembly (3/16): Extensibility

- RISC-V has base rule sets: RV32I (32-bit base integers) and RV64I (64-bit).

- There are optional add-ons (extensions) such as M for multiply/divide, A for atomics, C for compressed instructions.

- For this course, we build RV32IM = 32-bit base + multiply/divide.
Think: base toy set + extra pieces for more tricks.

### Slide 15 — RISC-V Assembly (4/16): RV32IM details

- PC is 32 bits (Program Counter = pointer).

- 32 registers, each 32 bits, named x0–x31.

- x0 is always zero (a drawer that always holds 0).

- Registers have nicknames (e.g., x2 is sp = stack pointer).

- Instructions are normally 32 bits; compressed 16-bit instructions also exist (C extension).
Translation: registers = little drawers; compressed = shorter recipe notes.

### Slide 16 — RISC-V Assembly (5/16): Instruction examples

Examples show different instructions and their meaning, e.g.:

- addi rd, rs1, imm12 → put (value in rs1 + the small number imm12) into rd.

- slli rd, rs1, imm12 → slide bits left (shift left).
Translate immediate (imm) = a small number written in the instruction. Shift = slide bits like beads on a stick.

### Slide 17 — RISC-V Assembly (6/16): Signed vs Unsigned

- Some instructions treat numbers as signed (can be negative) or unsigned (only positive).

- Example: slti compares signed values, sltiu compares as unsigned.
Translate: signed = with a “negative” mood; unsigned = only happy numbers.

### Slide 18 — RISC-V Assembly (7/16): Memory instructions

- Loads: lb (load byte), lh (load halfword = 16b), lw (load word = 32b).

  - lb/lh sign-extend → keep sign if the loaded small number is negative.

  - lbu/lhu zero-extend → pad with zeros instead.

- Stores: sb, sh, sw to write byte/half/word to memory.
Translate: load = take from shelf; store = put on shelf; extend = make a small item fit in a big drawer, keeping or not keeping the sign.

### Slide 19 — RISC-V Assembly (8/16): Memory granularity

- Byte-addressable = each address points to one byte (8 bits).

- You can’t directly address half-bits; the smallest address unit is 1 byte.
Think: each shelf spot holds one little box (one letter).

### Slide 20 — RISC-V Assembly (9/16): Memory alignment

- Memory hardware is often organized in bigger chunks (e.g., 4-byte words).

- Access is aligned if the address matches the chunk boundary (e.g., address divisible by 4 for 4-byte words).

- Misaligned accesses may require multiple reads or special handling.
Translation: alignment = neatly placing big blocks in aligned slots; misaligned = block sits across two slots and needs gluing.

### Slide 21 — RISC-V Assembly (10/16): Handling misaligned accesses

Ways to handle misalignment:

- Disallow and crash (simple hardware, not nice).

- Hardware handles it transparently (slower, more complex).

- Trap to software (ask software to fix) — simple hardware but big cost when it happens.
Compilers usually try to avoid misaligned accesses.

### Slide 22 — RISC-V Assembly (11/16): Endianness

- Endianness = order of bytes in multi-byte numbers.

  - Big-endian = big byte first (sensible order to some).

  - Little-endian = little byte first (RISC-V usually uses little endian).
Translation: order of packing; think: whether left shoe or right shoe is stored first.

### Slide 23 — RISC-V Assembly (12/16): Why little endian?

- Little-endian helps some pointer operations and became popular with x86.
Translate: it’s a choice driven by earlier computers; it makes certain operations free/cheap.

### Slide 24 — RISC-V Assembly (13/16): Jumps

- jal x1,offset = jump and link: go to PC+offset and write PC+4 into x1 (save return address).

- jalr x1,offset(x2) = jump to address x2+offset and save PC+4 into x1.

- Jump to x0 is just a plain jump (no link).
Translation: jal = jump and remember where you came from (like calling a friend and saving where to return).

### Slide 25 — RISC-V Assembly (14/16): Branches

- Branches compare registers and jump if condition holds, e.g., beq x1,x2,offset (branch if equal).

- There are signed and unsigned variants (e.g., bltu, bgeu).
Translation: branch = “if this is true, take that road; otherwise keep going.”

### Slide 26 — RISC-V Assembly (15/16): Example program (array sum)

Assembly example to add a list:

- Load base address, length, loop: load element, add to sum, increment pointer, decrement counter, repeat.
Translation: like counting candies in a bowl, one by one, until there are none left.

### Slide 27 — RISC-V Assembly (16/16): Summary (notes)

- There are compressed and variable length instructions historically; RISC-V standardizes many choices.

- Registers are often overwritten frequently.

- There are many instructions; we use the important ones for the datapath.

### Slide 28 — Outline (reminder)

Next we enter RV32I Datapath. (Reminder slide)

### Slide 29 — RV32I Datapath (1/13): Unified vs Split memory

- Unified memory = same bookshelf for program instructions and data.

- Harvard architecture = separate shelves for instructions and data.
Translate: unified = one bookshelf for both recipe and ingredients; Harvard = two different bookshelves.

### Slide 30 — RV32I Datapath (2/13): Example instruction set used

We’ll focus on a few instructions to build the datapath: lui, addi, add, beq, lw.
These let us show main datapath features without all surprises.

### Slide 31 — RV32I Datapath (3/13): Start with fetch

- Begin with PC and instruction memory.

- Add a +4 incrementer to compute the default next PC (PC + 4 for next instruction).
Translation: PC +4 = move to next page in the instruction book.

### Slide 32 — RV32I Datapath (4/13): First instruction: LUI

- lui x1,imm = load upper immediate: place a large constant into a register.

- Datapath needs the register file and a shifter (to put the immediate into the top bits).
Translate: shifter = sliding tool that moves the pieces to the right place.

### Slide 33 — RV32I Datapath (5/13): Next instruction: ADDI

- addi x1,x2,imm = x1 = x2 + imm.

- Need ALU and sign-extender (to make a small immediate into a full 32-bit number).

- Write either LUI result or ALU result to the register file.
Translate: sign-extender = stretcher that pads a small number to fit the big drawer.

### Slide 34 — RV32I Datapath (6/13): Next instruction: ADD

- add x1,x2,x3 = sum of two registers.

- ALU inputs must choose register values or immediate; use a multiplexer (MUX) to pick.
Translate: MUX = a road switch that decides which road a cart should take.

### Slide 35 — RV32I Datapath (7/13): Next instruction: BEQ

- beq x1,x2,target: use ALU to check x1 == x2 (subtract and check zero).

- Need a new adder to compute branch target (PC + offset), and a PC MUX to choose next PC.

- Why imm12 << 1? Because branch immediates are stored shifted for instruction alignment.
Translate: branch target adder = calculator that figures where to jump.

### Slide 36 — RV32I Datapath (8/13): Next instruction: LW

- lw x1,imm(x2): load word — compute address with ALU (x2 + imm), go to data memory, read value, write into register file.

- Writeback uses a MUX to choose ALU result vs loaded memory value.
Translate: compute address = find correct bookshelf spot to fetch the book.

### Slide 37 — RV32I Datapath (9/13): Immediate decoding

- Different instruction types store immediate fields in different bit positions (I, S, B, U, J types).

- This odd layout helps simpler wiring and smaller fanout in hardware.
Translate: immediates are packed on the instruction page in clever ways to make the internal wiring simpler.

### Slide 38 — RV32I Datapath (10/13): Open-source cores

- There are many small open cores (NERV, Sodor, picorv32, SERV). They are good references to see real designs but are often more complex.
Translate: open-source core = community toy kit you can look at to learn how others built theirs.

### Slide 39 — RV32I Datapath (11/13): What is control?

- Control = MUX selectors and write-enables (which MUX positions, whether to write to a register).

- Control must be specified per instruction, usually from opcode, funct3 and funct7 fields.
Translate: control = boss signals telling which roads to open and which drawers to write.

### Slide 40 — RV32I Datapath (12/13): Control via ROM (simple approach)

- ROM = read-only memory that stores control bits per opcode (like a look-up table).

- Addressed by opcode, returns control bit pattern (e.g., which MUX setting, RegWrite enable).
Translate: ROM = a cheat sheet that maps instruction type to the traffic-light settings.

### Slide 41 — RV32I Datapath (13/13): Control: logic (practical approach)

- Real machines have hundreds of control signals; ROM can be slow or large.

- Use logic gates (combinational logic / random logic) to compute control signals quickly from instruction bits.

- Some signals simplify to small checks (e.g., PCin = is_beq?).
Translate: logic = a set of simple yes/no doors that compute the correct traffic lights quickly.

### Slide 42 — Outline (reminder)

Next up: Register File — details.

### Slide 43 — Register File (1/10): Introduction

- Register file = M words of N bits each; here, 32 × 32 bits.

- Supports read/write with ports (two read ports, one write port in RV32I).
Translate: register file = many tiny drawers that the CPU can open quickly.

### Slide 44 — Register File (2/10): Decoder

- Decoder converts a binary address into a “one-hot” signal (only one line true).
Translate: decoder = the pointing finger that chooses exactly one drawer out of many.

### Slide 45 — Register File (3/10): Interfacing

- Inputs: RS1, RS2 (which registers to read), RD (which register to write), WE (write enable), RDestVal (value to write).

- Outputs: RSrc1Val, RSrc2Val (values read from RS1 and RS2).
Translate: interface = the set of inputs/outputs that tell which drawers to open and what to put inside.

### Slide 46 — Register File (4/10): Four registers (example)

Shows a small example with 4 registers to explain structure.
Translate: a small drawer cabinet example to show how decoder & MUX work.

### Slide 47 — Register File (5/10): Read port #1

- Each register’s output goes into a big MUX; RS1 selects which register connects to RSrc1Val.
Translate: MUX picks which drawer’s value to send to the calculator.

### Slide 48 — Register File (6/10): Read port #2

- Same as read port #1, but for RS2. Two independent readers mean the CPU can read two drawers at once.
Translate: two kids can open two different drawers at the same time.

### Slide 49 — Register File (7/10): Write port

- The write value is broadcast to all registers but only the one selected by the decoded RD and write-enable will actually latch the value.

- If you needed two write ports, hardware gets more complex.
Translate: all drawers hear the new value, but only the chosen drawer opens and takes the value.

### Slide 50 — Register File (8/10): Verilog example (interface)

- Shows a Verilog module header listing inputs/outputs: rd, rd_data, rs1, rs1_data, etc.
Translate: Verilog = “a language for describing the chip layout with text — like drawing the factory with words.”

### Slide 51 — Register File (9/10): Verilog example (implementation)

- Shows how registers are stored in an array, x0 is set to zero, reading uses the index, write on clock edge with we (write enable).
Translate: always @(posedge clk) = “on each tick of the clock, update the drawers if allowed.”

### Slide 52 — Register File (10/10): Memory vs Register File differences

- Register file: few words (<256), many ports (can read/write several at once), implemented for speed.

- Memory: many words (>1024), fewer ports (usually 1–2), implemented differently (cheaper per bit).
Translation: drawers are like many tiny cubbyholes you can open fast; memory is a big shelf with fewer doors to reach in.

### Slide 53 — Thank you / final slide

This slide ends the lecture and reminds: the material is for class attendants only.
Key takeaways: how instructions are fetched/decoded/executed, how datapath & control work, and how register files and memory differ.

## Short review cheat-sheet (kid version) — one line per big idea

- Processor = tiny brain that follows recipe pages.

- PC (Program Counter) = arrow pointing to the next recipe line.

- Register = little super-fast drawer.

- Memory = big bookshelf.

- ALU = tiny calculator.

- Datapath = roads inside the brain where numbers travel.

- Control = traffic lights and boss rules that guide traffic.

- MUX (Multiplexer) = a road switch that chooses which input to use.

- Decoder = finger that points to exactly one drawer.

- ROM = prewritten cheat sheet mapping instructions to control signals.

- Combinational logic = instant compute blocks (no memory).

- Sequential logic = things that remember between beats (registers, PC).

- Single-cycle = one instruction per long beat; simple but must wait for slowest instruction.

- Pipelining = split into stages so many instructions can be in flight at once (faster, needs hazard fixes).

- Hazards = when instructions get in each other’s way (data hazard, control hazard, structural hazard).

- Forwarding = hand fresh results forward; stalling = insert pauses; flush = throw away wrong work and restart.

If you’d like, next I can:

- Turn this cheat-sheet into a single Word file or printable PDF (with the same kid-friendly slide-by-slide text), or

- Expand any particular slide into a longer kid-story with examples and tiny diagrams (for slides like 31–36 where datapath wiring and branch logic get tricky), or

- Convert the content into flashcards (one slide per card) for quick study.

Which of these would you like me to do next?